ANDROID UHF API

1Package and library import project

The UHF function is wrapped in uhf.jar with com.android.hdhe.uhf.reader.Uhfreader.

2 Function instruction

2.0.1  getInstance()

Function: Get the UhfReader instance and call the UHF encapsulation instruction

Parameter ：No

Back：UhfReader instance

2.0.2 setOutputPower(int value)

Function ：Set the transmit function (adjust the read tag distance)

Parameter ：int value，value range 14-26 14dBm~26dBm

Back：boolean flag,true Set success，false Set failed

2.0.3 inventoryMulti()

Function: multiple calls inside the chip, suitable for multi-label environment (saved 10,000 times, and automatically stopped, please call again)

Parameter：No

Back：List<byte[]> list,when list is not null，Returns the inventory list，when list is null，No label or the label is not in the antenna area.

2.0.4 stopInventoryMulti()

Function: Stop the multi-label inventory

Parameters: No

Back: No

2.0.5 inventoryRealTime()

Function: real-time inventory, for a single execution instruction, inventory can be called repeatedly, to achieve multi-label inventory

Parameters: No

Back：List<byte[]> list,when list is not null，Returns the inventory list，list为null，when list is null，No label or the label is not in the antenna area.

2.0.6 selectEPC(byte[] epc)

Function: Select a label to be called before you read or write it。

Parameter ：byte[] epc,The EPC data for the label

Back: No

2.0.7 unSelectEPC()

Function: Deselect the label and should be called when reelecting the label or inventory。

Parameters: No

Back：int, Execution correctly when for 0，Others are wrong。

2.0.8 readFrom6C(int memBank, int startAddr, int length, byte[] accessPassword)

Function: Read the label storage data, the data read in word。

Parameter ：int memBank storage area RESEVER is 0，EPC field 1，TID field 2，USER field 3；

int startAddr start address（cannot be too large, depending on the size of the data area）；

int length  Read the data length，Its unit is word(1word = 2bytes);

byte[] accessPassword password 4 bytes

Back：byte[] data;when data is not null，data is for the data area，when is null, The read data failed，returns a byte of the error code

2.0.9 writeTo6C(byte[] password, int memBank, int startAddr,  int dataLen, byte[] data)

Function: Write label storage data (TID field not writable)，Write the data in word，if not enough ,to fill 0。

Parameter ：byte[] Password Password 4 bytes

int memBank storage area RESEVER field is 0，EPC field 1，TID field 2，USER field 3；

int startAddr start address（cannot be too large, depending on the size of the data area）；

int dataLen

Read the data length，Its unit is word(1word = 2bytes) ;

byte[] data data back：boolean ,true Write success，false Write failed

2.0.10 setFrequency(int startFrequency, int freqSpace, int freqQuality)

Function: Adjust the operating frequency (not recommended)。

Parameter ：int startFrequency

int freqSpace

int freqQuality ；

Back：int , If return 0，Settings are successful，Others fail

2.0.11 setWorkArea(int area)

Function ：Set the Area Frequency (recommended)。

Parameter ：int area；

area =  1 China 2

area = 2 American-Standard

area = 3 European standard

area = 4 China 1

area = 5 Korea

Back：int ,If return 0，Settings are successful，Others fail